# Quiz II – CE 5972 Optimisation Techniques in Transportation Engineering

Name:								Roll No.:

- Transportation Problem: Sree Krishna Bricks operates two brick manufacturing plants in Medavakkam that cater to five client construction sites across Chennai. The table below presents supply capacities of the plants and demand requirements of the sites (in tonnes). Additionally, it details the cost of transporting a tonne of bricks between each plant and site (in ₹k). Determine the shipping plan for the company that minimises the transportation costs while satisfying the supply and demand constraints.

| From/To | Site 1 | Site 2 | Site 3 | Site 4 | Site 5 | Supply |
| --- | --- | --- | --- | --- | --- | --- |
| Plant 1 | 15 | 11 | 18 | 12 | 14 | 80 |
| Plant 2 | 16 | 17 | 21 | 10 | 13 | 70 |
| Demand | 45 | 20 | 30 | 30 | 25 |  |

  - Define the decision variables. (1)

  - Formulate the objective function for this problem. (2)

  - Formulate the supply constraints. (2)

  - Formulate the demand constraints. (5)

  - Formulate the domain constraints. (1)

- Vehicle Routing Problem: We have a directed graph G=(N,A); N=d∪C, where d represents the depot node, C represents set of customer nodes, and A represents the set of arcs connecting these nodes. The depot has an associated set of electric delivery vehicles V, wherein each vehicle has a capacity qv, range lv, and a traversal cost of πo per km on arc i,j∈A spanning a length of length dij. Further, each customer node c∈C has a demand qc that must be fulfilled from the depot nodes via one of its electric delivery vehicles. For the given problem,

  - Define the decision variables. (2)

  - Formulate the objective function. (2)

  - Define head and tail set function. (2)

  - Formulate the following constraints

    - customer service constraint (1)

    - flow conservation constraint (2)

    - vehicle use constraint (assume vehicle can perform at most one delivery tour) (1)

    - fleet size constraint (1)

    - vehicle load balance constraint (3)

    - vehicle load boundary condition (1)

    - vehicle capacity constraint (1)

    - vehicle range constraint (1)

    - domain constraints (2)